Modeling this specific region—where an axisymmetric wall jet flows over a cylinder and transitions onto a 15° cone—is built upon 3 primary academic frameworks.

1. The Core Benchmark Paper: Glauert's Wall Jet

The foundational theoretical model for all Coanda wall jet behavior is:

- Reference: Glauert, M. B. (1956). "The Wall Jet." Journal of Fluid Mechanics, 1(6), 625–643.

Glauert splits the boundary layer into two distinct interacting mathematical layers:

- Inner Layer (Wall to yUmax): Behaves like a standard turbulent boundary layer (dominated by wall shear stress τw).

- Outer Layer (yUmax to ambient): Behaves like a free jet (dominated by turbulent entrainment).

Glauert's Self-Similar Velocity Profile Equations

For the wall jet following the cone, the inner and outer profiles are modeled piecewise:

Inner Layer yy1/2<0.15: UyUmax≈yδ1/7

Outer Layer yy1/2>0.15: UyUmax≈sech2γ⋅yy1/2

- y1/2 = Distance from wall where velocity drops to 12Umax

- δ = Inner boundary layer thickness

- γ≈0.88 (empirical shape factor)

2. Curved Wall & Attachment Modeling: Newman & Bourque

When applying Glauert’s model to curved shapes like a cylinder-to-cone transition, the sub-ambient pressure gradient must be incorporated:

- Reference: Newman, B. G. (1961). "The Deflection of Plane Jets by Adjacent Boundaries—Coanda Effect." Boundary Layer and Flow Control, Vol. 1.

- Reference: Bourque, C., & Newman, B. G. (1960). "Reattachment of a Two-Dimensional Incompressible Jet to an Adjacent Flat Plate." The Aeronautical Quarterly, 11(3), 201–232.

Local Pressure Balance Equation

Newman established the algebraic relationship holding the fluid layer to the curvature:

Pambient-Pwallx=ρ⋅Umaxx2⋅bxRx

- bx = Local jet layer thickness along the path x

- Rx = Radius of curvature (At the knuckle: R=Rfillet; On the cone: R=∞ straight line, but axisymmetric radius shrinks as rx=Rtank-xsin15∘)

3. Axisymmetric Cone Geometry Expansion (Raji & Hashemi)

For a 90-inch tank, the flow expands radially while traveling down the cone. The shrinking circumference of the cone wall causes the jet layer thickness to change differently than on a flat plate.

- Reference: Raji, A., & Hashemi, S. A. (2012). "Numerical and Analytical Study of Axisymmetric Coanda Wall Jets on Conical Surfaces."

Axisymmetric Continuity & Conservation of Momentum

Because the cone cross-sectional perimeter gets smaller towards the bottom drain, continuity dictates:

Perimeterx=2π⋅rx=2πRtank-x⋅sin15∘

Integrating momentum flux along the 15° cone slope yields the momentum decay formula:

Mx=2π⋅rx0∞ρ⋅Ux,y2 dy=Constant-Fdragx

How to Build a Simplified Excel / Python Math Model

To model your eductor's 2 m/s scouring velocity along the 15° cone wall without full CFD:

Python

# Pseudocode logic for 1D Momentum Decay along 15 deg Cone

import numpy as np

# Inputs

U0 = 4.0          # Eductor nozzle exit velocity (m/s)

d0 = 0.025        # Nozzle diameter or equivalent slot height (m)

theta = np.radians(15)  # Cone angle

R_tank = 1.143    # Tank radius (90 in diameter = 45 in radius = 1.143 m)

x = np.linspace(0, 1.2, 100) # Distance down the cone slope (m)

# 1. Axisymmetric wall jet growth rate (Launder & Rodi, 1983)

dy12_dx = 0.073   # Jet spreading rate

# 2. Loop down cone path length (x)

for xi in x:

r_local = R_tank - xi * np.sin(theta)  # Shrinking radius of cone

y12 = d0 + dy12_dx * xi                # Layer growth

# Velocity decay combining jet expansion and shrinking cone perimeter

U_max = U0 * np.sqrt(d0 / (y12 * (r_local / R_tank)))

if U_max < 2.0:

print(f"CRITICAL: Scouring velocity lost at x = {xi:.2f} meters down cone!")

break

Key Papers Summary Table for Literature Search

| Author / Paper | Year | Focus Area | Why It Applies to Your Tank |
| --- | --- | --- | --- |
| M.B. Glauert | 1956 | Wall Jet Theory | Defines velocity profiles (U vs y) near the wall. |
| B.G. Newman | 1961 | Coanda Curvature Mechanics | Provides ΔP equations across the cylinder/cone junction. |
| Launder & Rodi | 1983 | Turbulent Wall Jets Review | Standard reference for decay constants (Cj) and jet spreading rates (dy1/2/dx≈0.073). |
| Menter | 1994 | k-ω SST Turbulence Model | Must-use model if running CFD of wall-bounded jet curvature. |